APPLICATION NOTE for

SMIA95-5M Auto Focus camera module

Module model name : TCM8341MD

Sensor model name : ET8EK8-AS

Ver. 2.30b

CONTENTS

a. Overview	3

b. System diagram	3

c. INTERFACE TERMINAL DESCRIPTION	4

1. Power supply control	5

1-1 Power up/down sequence	5

1-2 Timing chart of POWER ON	5

1-3 Initial command settings	6

1-4 Timing chart of POWER OFF	7

1-5 Software Stand-by	7

1-6 Image data of sensor restart	7

2. IIC control I/F	8

3. Register map	9

4. Sensor version information	12

5. Output-mode	12

6. Start pixel information	14

7. Data output specification	14

8. Command effect timing	15

9. PLL and Frame rate setting when CCP2 output	15

10. Picture size and View finder setting	17

10-1 Picture size setting	17

10-1-1 Output format setting	17

10-2 VGA view finder with zoom setting	17

10-3 How to digital zoom in case of VGA	17

11. Analog gain and Digital gain setting	18

11-1 Analog gain	18

11-2 Digital gain	19

13. Preset gain and White balance gain	20

14. Electrical shutter	20

15. Data format of CCP2 Output	21

16. CCP2 Skew Adjustment	24

17. Revision History	25

a. Overview

This document specifies a very compact size of 1/2.5” 5M pixel auto-focus camera module compatible with SMIA95 size.

- SMIA95 size auto focus camera module suitable for the main stream cameras

- Superb picture quality by employing Toshiba’s best-in-class 5 Mega-pixel CMOS sensor with 2.2 um pixel.

- Good object distance coverage by auto-focus actuator.

- Mitigated motion artifact for downsized picture by enabling 30 fps operation .

Table 1 General Overview

| Area | Item | Type |
| --- | --- | --- |
| Sensor (model name ET8EK8-AS) | Maximum Resolution | 2592 (H) x 1968 (V) |
|  | Color filtering type | Bayer |
|  | Optical format | 1/2.5 “ |
|  | Pixel size | 2.2 um |
| Optical System | F number | F2.8 |
|  | Focusing type: fixed / auto focus | Auto Focus |
| Module System | Actuator Driver | Embedded inside camera Module |
| Data interface | Image data bus | CCP2 sub LVDS |
|  | Supported output formats | 10 bit RAW Bayer |
|  | Control bus | IIC bus |
| Mechanical specification | Module size | 9.5 (D) x 9.5(W) x 8.0(H) mm |
|  | PAD count | 16 |

VCM

(1) Operating Condition

- Actuator type:               Voice Coil Motor

- Rated Current:               80mA

- Absolute Maximum Current :  100mA

(2) Performance and Characteristics

- Stroke :                     min 0.31mm ( at all directions, when applied 80mA)

- Starting Current :             min 10mA ( at horizontal direction )

b. System diagram

1/2.5”5 M sensor

Image processor

VANA(2.8)V

VDIG (1.8V)

BB engine

XSHUTDOWN

CCP2

IIC

2

2

4

2

VCM actuator

Driver-IC AD5820

Extclk

VBat

2

c. INTERFACE TERMINAL DESCRIPTION

Bottom view

Pad 1

Pad 9

Pad 16

Pad 8

VCAP

VANA

XSHUTDOWN

EXTCLK

SCL

SDA

AGND

DATA+

DATA-

VDIG

DGND

DGND

CLK+

CLK-

Vbat

GNDVbat

| Pad No. | Signal name | Signal description |
| --- | --- | --- |
| 1 | Vbat | Battery voltage supply Can not be connected to VANA directly |
| 2 | VCAP | Open or can be connected to GND via a capacitor |
| 3 | AGND | Ground for VANA |
| 4 | VANA | Voltage source to camera module |
| 5 | XSHUTDOWN | Power down control |
| 6 | ExtClk | External clock signal from BB engine. |
| 7 | SCL | I2C clock |
| 8 | SDA | I2C data line |
| 9 | DGND | Digital GND |
| 10 | CLK- | CCP2 bus: Differential clock output, negative polarity signal |
| 11 | CLK+ | CCP2 bus: Differential clock output, positive polarity signal |
| 12 | VDIG | Digital GND |
| 13 | DATA- | CCP2 bus: Differential data output, negative polarity signal |
| 14 | DATA+ | CCP2 bus: Differential data output, positive polarity signal |
| 15 | DGND | Digital GND |
| 16 | GNDVbat | Ground for Battery voltage supply |

1. Power supply control

1-1 Power up/down sequence

Operating state

EXTCLK ON

XSHUTDOWN ON

XSHUTDOWN OFF

IIC commands

(*1) VANA(2.8V) ON

VDIG(1.8V) ON

IOVDD(2.8V or 1.8V) ON

EXTCLK OFF

Power down state

(*2)VANA(2.8V) OFF

VDIG(1.8V) OFF

IOVDD(2.8V or 1.8V) OFF

Order of input about IIC command.

(1)LSTB (Add:0x126C bit8 = 1, default is 0)

(2) Another commands.

(3) SLEEP(Add:0x1252 bit8 = 1, default is 0)

Fig1-1: Power up/down sequence

1-2 Timing chart of POWER ON

(*1)VANA and VDIG and IOVDD may rise in any order.

The rising separation can vary from 0ns to Indefinite

T6≧5000EXTCLK

Commands Input

T1≧0

T2≧0

T3≧1(msec)

VANA

(2.8V)

VDIG18

(1.8V)

IOVDD

(2.8V or 1.8V)

XSHUTDOWN

EXTCLK

SCL/SDA

Fig1-2: Timing chart of POWER ON

1-3 Initial command settings

When power on, please set these commands only one time.

This initial setting is below status. Please also refer to command settings section.

(1) Serial out

(2)RAW10

(3)5M output, w/o embedded data, 640Mbps

Below file is initial command settings and changing history of setting based on mode2 (5M full output)

And also we recommended input order of register settings about both sensors.

This version is ver1.7

1-4 Timing chart of POWER OFF

T7≧0

T9≧0

T8≧0

VANA

(2.8V)

(*2)VANA and VDIG and IOVDD may fall in any order.

The falling separation can vary from 0ns to Indefinite

VDIG

(1.8V)

IOVDD

(2.8V or 1.8V)

XSHUTDOWN

EXTCLK

Fig1-3: Timing chart of POWER ON

1-5 Software Stand-by

If changing to software standby status, SLEEP set from H to L (Address: 0x1252, D7).

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1252 | D7 | SLEEP | 0d: Software Standby status 1d: Streaming (Active) |

Software standby status

|  | CCI I/F | Sensor | CCP2 | ISP | VCO |
| --- | --- | --- | --- | --- | --- |
| Software standby(*1) | Active | Stop | Hi-Z | Stop | Stop |
| Operation | Active | Active | Active | Active | Active |

(*1)-Software standby is from High level of Xshutdown to first I2C command input

-During after input 30h (Add: 0x1252) and before input B0h (Add: 0x1252).

1-6 Image data of sensor restart

When sensor restarts, Normal image can get 2 frames after.

# 2. IIC control I/F

Sub address is 2 bytes, and first data is MSB, last data is LSB.

Write mode

MSB 7bit         MSB 8bit　      MSB 8bit 　　 MSB 8bit

Read mode

MSB 7bit         MSB 8bit　    MSB 8bit 　　   MSB 8bit　　　 MSB 8bit

Host Command   　     　     Camera

S : Start condition ,  P : End condition ,  A : Acknowledge,  Ā : not Acknowledge

Start condition , End condition                    Bit Transfer

Acknowledge　　　　　　　　　　　　　　　　　  Not Acknowledge

Slave Address ( 2 table can be selected )

Sensor

| A6 | A5 | A4 | A3 | A2 | A1 | A0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1/0 |

7bit Slave address is used.

The system conforms to the I2C Standard Specification defined by Philips.

Actuator driver

| A6 | A5 | A4 | A3 | A2 | A1 | A0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1/0 |

# 3. Register map

Gray mesh area is not defined. Default value is recommended in white area.

# 4. Sensor version information

This sensor can be recognized if reading below address data.

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1200 | D7-D0 | VER_NUM[7:0] | VER_NUM[15:0] =01h in case of #1Sensor. |
| 0x1201 | D7-D0 | VER_NUM[15:8] |  |

# 5. Output-mode

This sensor has 14 modes of output. ( 7 type serial and 7 type parallel)

Output               Serial              Raw10

Raw12

Raw8

DPCM 10bits to 8bits

DPCM 12bits to 8bits

DPCM 10bits to 6bits

DPCM 12bits to 6bits

Parallel　　　　　   12bit

10bit

8bit

DPCM10bits to 8bits

DPCM12bits to 8bits

DPCM10bits to 6bits

DPCM12bits to 6bits

Table5-1: Explanation of Output-mode parameter

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x125D | D7 | CCP_LVDS_MODE | 0h : Parallel output (default) 1h : Serial output |
|  | D3 | USE_PRED2 | The predictor for DPCM compression. 0h : use predictor1 for DPCM 10bits to 8bits 1h : use predictor2 for other DPCMs (default) |
|  | D2-D0 | CCP_COMP_MODE[2:0] | (1) Serial output case 0h RAW10 1h RAW12 2h RAW8 3h DPCM 10bits to 8bits 4h DPCM 12bits to 8bits (default) 5h DPCM 10bits to 6bits 6h DPCM 12bits to 6bits (2) Parallel output case 0h 12bit 1h 10bit ( from LSB to 2nd LSB output “L” level) 2h 8bit ( from LSB to 4th LSB output “L” level) 3h DPCM 10bits to 8bits ( from LSB to 4th LSB output “L” level) (default) 4h DPCM 12bits to 8bits ( from LSB to 4th LSB output “L” level) 5h DPCM 10bits to 6bits (from LSB to 6th LSB output “L” level) 6h DPCM 12bits to 6bits (from LSB to 6th LSB output “L” level) |

How to set DPCM

[1] DPCM 10bits to 8bits

- CCP_COMP_MODE = 3d

- USE_PRED2 = 0d

- MRCK = SPCK = 1/8 and MRCK=1/8,SPCK=1/7 and MRCK=1/8,SPCK=1/6

[2] DPCM 12bits to 8bits

- CCP_COMP_MODE = 4d

- USE_PRED2 = 1d

- MRCK = SPCK = 1/8

[3] DPCM 10bits to 6bits

- CCP_COMP_MODE = 5d

- USE_PRED2 = 1d

- MRCK = SPCK = 1/6

How to set MRCK and SPCK,

Please refer to the Section 9. PLL and Frame rate Setting when CCP2 output.

# 6. Start pixel information

Start pixel information is as follows.

- B pixel (In case of No top/bottom flip and No right/left flip)

- Gb pixel (In case of No top/bottom flip and right/left flip)

- Gr pixel (In case of top/bottom flip and No right/left flip)

- R pixel (In case of top/bottom flip and right/left flip)

# 7. Data output specification

This sensor has dummy data at 1st line and end line.

All of output mode(e.g.binning mode/crop mode ) have dummy data.

5M full output case (No top/bottom flip and No right/left flip)

Image data

LS

FS

Dummy Data

LE

FE

Dummy Data

B

Gb

Gr

R

1966lines

1968lines

Dummy data specification

RAW10 case

0A 07 07 07 55 07 07 07 07 55 07　・・・・・・ 07 07 07 07 55

RAW8 and other format

0A 07 07 07　・・・・・・ 07

Dummy bit of multiple of 32 in serial output mode

Dummy data is without 0x00

With or without Dummy data can be selected by a resister. (Add:0x125E, D5. Resister name is

EMBEDDED_DATA.  0h:OFF(without dummy data) 1h:ON(with dummy data) ,default setting is 1h)

Fig7-1: In case of WITH dummy data           Fig7-2: In case of WITHOUT dummy data

Image data

LS

FS

Dummy Data

LE

FE

Dummy Data

B

Gb

Gr

R

1966lines

1968lines

1966lines

1968lines

Image data

LS

FS

LE

FE

B

Gb

Gr

R

# 8. Command effect timing

If changing parameter setting, input at period of imaging data. Setting parameter will be effected next frame.

About electrical shutter, will be effected 2 flames after.

In detail, Command latch timing from FE are following formula.

LTfFE(=Latch Timing from FE) = V blanking -7 (line)

These values depend on mode setting. Also refer to “column M” in a excel sheet at Page14

electrical shutter

W/o electrical shutter

FS

Image data

FE

FS

Image data

FE

FS

Image data

FE

LTfFE

LTfFE

Command latch

Command latch

Command setting

Fig8-1: Command effect timing

# 9. PLL and Frame rate setting when CCP2 output

(1) About CCP2 output case, please keep to the following relation of frequency

LVDSCK : MRCK = 8 : 1   (w/o DPCM 10bits to 6bits)

(2) Fastest frame rate will be needed, relation(1) and VCO_DIV=0d(x1 setting)

EXTCLK

Divider

CKREF_DIV [3:0]

Phase comparator

VCO

Divider

CKVAR_DIV [8:0]

Divider

VCO_DIV [3:0]

LVDSCK_DIV [3:0]

LVDSCK

Divider

Divider

SPCK_DIV [3:0]

SPCK

MRCK

1/8

MRCK_DIV [3:0]

SPCK: Sensor & ISP system clock before FIFO

MRCK: 1/8 of CCP2 data rate

Fig9-1: PLL circuit

Fig9-2: Relation between SPCK, MRCK and LVDSCK

Table9-1: Explanation of PLL parameter

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1238 | D3-D0 | CKREF_DIV[3:0] | Selection of reference clock for PLL 0d: 1/1, 1d: 1/2 , 2d: 1/3 , 3d: 1/4 ,,,, 15d: 1/16 |
| 0x1239 | D7-D0 | CKVAR_DIV [8:0] | Selection of divider clock for PLL 0d to 15d : N/A , 16d: 1/16 , 17d: 1/17 ,,,, 510d: 1/510 , 511d: 1/511 |
| 0x1238 | D7 |  |  |
| 0x123A | D7-D4 | VCO_DIV[3:0] | Selection of divider of VCO clock 0d: 1/1, 1d: 1/2 , 2d: 1/3 , 3d: 1/4 ,,,, 15d: 1/16 |
|  | D3-D0 | SPCK_DIV[3:0] | Selection of divider for Clock for LOGIC 0d: 1/1, 1d: 1/2 , 2d: 1/3 , 3d: 1/4 ,,,, 15d: 1/16 |
| 0x123B | D7-D4 | MRCK_DIV[3:0] | Selection of divider for Clock for LOGIC 0d: 1/1, 1d: 1/2 , 2d: 1/3 , 3d: 1/4 ,,,, 15d: 1/16 |
|  | D3-D0 | LVDSCK_DIV[3:0] | Selection of divider for LVDS clock 0d: 1/1, 1d: 1/2 , 2d: 1/3 , 3d: 1/4 ,,,, 15d: 1/16 |

Detail of PLL and Frame rate setting, please refer to below a excel sheet (table2).

This version is ver1.7.

Formula is as follows

VCO [MHz] = EXTCLK * CKVAR_DIV / 3

CCP2 [Mbps] = VCO / { ( LVDSCK_DIV + 1 ) * ( VCO_DIV + 1 ) }

spck [MHz] = VCO / { ( SPCK_DIV +1 ) * ( VCO_DIV + 1 ) }

Frame rate [fps] = { spck / ( IA_H * IA_V ) } * 1,000,000

( IA_H and IA_V : H and V values of Image area readout with blanking. )

# 10. Picture size and View finder setting

10-1 Picture size setting

Table10-1: Explanation of output format parameter

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1221 | D2-D0 | H_COUNT[10:0] | Selection of total horizontal clock count including blanking Please refer to excel sheet of table of Setting rage(*1) |
| 0x1220 | D7-D0 |  |  |
| 0x1223 | D4-D0 | V_COUNT[12:0] | Selection of total Vertical line count including blanking Setting rage is from 84d to 2730d. |
| 0x1222 | D7-D0 |  |  |
| 0x1225 | D7 | TBINV | ON/OFF of vertical inversion 0d: OFF(normal), 1d:ON (vertical flip) |
|  | D6 | RLINV | ON/OFF of horizontal inversion 0d: OFF(normal), 1d:ON (horizontal flip) |
| 0x121D | D6-D4 | H_SIZE[2:0] | Horizontal CROP mode 0d:1/6, 1d: 1/4, 2d: 1/3, 3d: 1/2, 4d: 2/3, 5d: 3/4, 6d-7d: no crop (=full) |
|  | D2-D0 | H_INTERMIT[2:0] | Horizontal digital scaling mode 0d:1/6, 1d:1/4, 2d: 1/3, 3d:1/2 4d-7d: 1/1(=full) |
| 0x121B | D6-D4 | PIC_SIZE[2:0] | Vertical CROP mode 0d:1/6, 1d: 1/4, 2d: 1/3, 3d: 1/2, 4d: 2/3, 5d: 3/4, 6d-7d: no crop (=full) |
|  | D2-D0 | MONI_MODE[2:0] | Horizontal pixel binning mode 0d:1/6, 1d:1/4, 2d: 1/3, 3d:1/2 4d-7d: 1/1(=full) |
| 0x1224 | D5-D0 | F_COUNT[5:0] | Selection of intermittent frame output mode 0d: 1/1, 1d: 1/2 , 2d: 1/3 , 3d: 1/4 ,,,, 63d: 1/64 |

Below excel sheet is table of H_count setting range.

If Night mode setting like a long exposure is needed, please increase value of V_COUNT (not slowing down of

vco/pixclk).

10-1-1 Output format setting

Please refer to a excel sheet at Page 11(table2)

10-2 VGA view finder with zoom setting

Please refer to a excel sheet at Page 11 (table3)

10-3 How to digital zoom in case of VGA

Please refer to a excel sheet at Page 11 (digital zoom page)

# 11. Analog gain and Digital gain setting

Table11-1: Explanation of Gain setting parameter

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x124A | D3-D0 | MAG[11:0] | Analog gain setting 20h: 0dB, 40h: 6dB, 80h: 12dB, 100h: 18dB(max recommended) |
| 0x1249 | D7-D0 |  |  |
| 0x124D | D1-D0 | MDG[9:0] | Gain setting of digital gain 0h: x1, 3FF: x 2 (Normal mode) ( **Depend on setting of DGM_MODE) |
| 0x124C | D7-D0 |  |  |
| 0x124D | D7-D6 | DGM_MODE[1:0] | 0d : from x1 to x2 1d : from x0 to x2 2d : from x1 to x3 3d : from x0 to x4 |

11-1 Analog gain

Analog gain can be changed by MAG [11-0]. (Add:0x1249 Data:D7-D0 and Add:0x124A Data:D3-D0)

Analog gain [dB] = 20*log10 (setting value(decimal) /32)

Setting range is from 0x020 to 0x100.

12[dB]

8[dB]

6[dB]

0[dB]

18[dB]

11-2 Digital gain

Digital gain can be changed by MDG [9-0]. (Add:0x124C Data:D7-D0 and Add:0x124D Data:D1-D0)

Changing DGM_MODE(Add:124D D7-D6), characteristics of digital gain can be changed.

DGM_MODE=0d

Digital gain[dB] = 20*log10(1/1023*Setting value(hex)+1)

DGM_MODE=1d

Digital gain[dB] = 20*log10(2/1023* Setting value(hex))

DGM_MODE=2d

Digital gain[dB] = 20*log10(2/1023* Setting value(hex)+1)

DGM_MODE=3d

Digital gain[dB] = 20*log10(4/1023* Setting value(hex))

0[dB]

6[dB]

9[dB]

12[dB]

# 13. Preset gain and White balance gain

Table13-1: Explanation of White balance parameter

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1435 | D7-D0 | PWB_RG[7:0] | Pre-set gain for Red pixel (00h:x1, FFh:x3) |
| 0x1436 | D7-D0 | PWB_GRG[7:0] | Pre-set gain for G pixel in Red line. (00h:x1, FFh:x3) |
| 0x1437 | D7-D0 | PWB_GBG[7:0] | Pre-set gain for G pixel in Blue line. (00h:x1, FFh:x3) |
| 0x1438 | D7-D0 | PWB_BG[7:0] | Pre-set gain for Blue pixel（00h:x1, FFh: x3) |
| 0x124E | D1-D0 | WB_RG[9:0] | Gain setting of B digital gain for white balance 0h: x0, 100h: x1, 200h: x2, 3FFh: x 4 |
| 0x124F | D7-D0 |  |  |
| 0x124E | D3-D2 | WB_GG[9:0] | Gain setting of G digital gain for white balance 0h: x0, 100h: x1, 200h: x2, 3FFh: x 4 |
| 0x1250 | D7-D0 |  |  |
| 0x124E | D5-D4 | WB_BG[9:0] | Gain setting of B digital gain for white balance 0h: x0, 100h: x1, 200h: x2, 3FFh: x 4 |
| 0x1251 | D5-D4 |  |  |
| 0x1439 | D7 | DIG_PWB_SW | Switch of Preset-White-balance (0d:disable / 1d:enable) |

# 14. Electrical shutter

Table14-1: Explanation of Electrical shutter parameter

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1244 | D7-D0 | MES[15:0] | Setting of electrical exposure time 1h: 1H ~ FFFFh: 65535H ( max) |
| 0x1243 | D7-D0 |  |  |

max exp time(MES) = VCOUNT value (=Total line numbers ) - 4line

# 15. Data format of CCP2 Output

8 byte of synchronization codes are attached at the start and the end of each frame and each line.

|  | Code | Value |
| --- | --- | --- |
| SOL | Line start code | FFH 00H 00H 00H |
| EOL | Line end code | FFH 00H 00H 01H |
| SOF | Frame start code | FFH 00H 00H 02H |
| EOF | Frame end code | FFH 00H 00H 03H |

<Data structure in one vertical scan for CCP2 output>

To detect possible errors in transmission, a checksum is calculated over each data line. The checksum is realized as 16-bit CRC. The generator polynomial is x^16+x^12+x^5+1. The checksum sequence is then padded with 16-bit padding sequence (101010 ---10) to fulfill the 32-bit word alignment requirement in the CCP link. The transmission of checksum is illustrated in the figure below.

Data description in each byte in one line (LSB first expression)

<Bit by bit structure>

Output mode of sensor raw data

10 bit sensor raw data are divided into upper 8 bit and lower 2 bit to send 10 bit depth data in 8 bit data scheme of CCP serial output. The following procedure and data arrangement is implemented to avoid 00h and FFh appearance in each 8 bit data.

(1)

Sensor raw data range is compressed from 0 -1023 to 4 -1023 .

Namely 0-3 data are set to 4 .

This compaction does not affect the picture quality, because ,

for example, the black level is set to 64 level .

(2)

10 bit data ( 4 - 1023) is separated into upper 8bit and lower 2 bit.

The upper 8 bit has never all "0"  value.

The four pixel data with 10 bit  are arranged into the following five bytes.

1

upper 8 bit of pixel A

2

upper 8 bit of pixel B

3

upper 8 bit of pixel C

4

upper 8 bit of pixel D

5

4 of lower 2 bit for 4 pixels

When all bits of No.5 byte are 0,  00h is replaced by 10h.　In the　LSB　first form, “00001000” is used in conformity with CCP2 specification.

(3)

The host rearranges the five bytes to 4 pixel data with 10 bit.

# 16. CCP2 Skew Adjustment

I2C command at 0x1269 address is used to adjust the delay of data and strobe signal in CCP2 to compensate skew between data and strobe.  CRC data is checked for a certain pattern data by changing the data at address 0x1269 as 40h->30h->20h->10h->00h->01h->02h->03h->04h to find out OK zone. After that 0x1269 is set corresponding to the center of the OK zone.

| Address | Data | Bit Symbol | Description |
| --- | --- | --- | --- |
| 0x1269 | D7-D4 | CCP2SDLY[3:0] | CCP2 strobe signal delay adjustment 0h: No delay, 1h: 0.1nsec delay, 2h: 0.2nsec delay, ~ Fh: 1.5nsec delay |
|  | D3-D0 | CCP2DDLY[3:0] | CCP2 data signal delay adjustment 0h: No delay, 1h: 0.1nsec delay, 2h: 0.2nsec delay, ~ Fh: 1.5nsec delay |
| 0x1268 | D7 | CCP2STP_X | CCP2 output ON/OFF 0h: ON(=operate), 1h: OFF(=stop) |
| 0x1263 | D4 | CCP2CRCSW | CRC data output ON/OFF 0h: OFF, 1h: ON(CRC data are added) |

# 17. Revision History

Revision           Date                Comments

Ver2.30a           21th Dec, 2008       Draft

Ver2.30b           11th May, 2009       Following yellow hatched items are added.

Sensor/Module model name (P1,P3)

Data format of CCP2 Output(P21)

CCP2 Skew Adjustment(P24)